Численное моделирование массотеплопереноса в воздушном зазоре между пластинами методом ICFD

Котов Андрей Андреевич1 				kotovaa1@student.bmstu.ru

Чередниченко Александр Всеволодович1 		alexvche@bmstu.ru

1 МГТУ им. Н.Э. Баумана, Москва, Россия

Аннотация. В работе представлены результаты численного исследования теплообмена в воздушном зазоре между двумя параллельными медными пластинами. Нижняя пластина нагрета, верхняя охлаждается за счет конвекции. Исследование выполнено в программном комплексе LS-DYNA с применением метода ICFD (Incompressible Computational Fluid Dynamics) для моделирования течения воздуха и теплопереноса. Приведены постановка задачи и математическая модель. Основное внимание уделено описанию метода ICFD, его ключевым алгоритмам решения уравнений Навье-Стокса для несжимаемой жидкости. Проведена серия вычислительных экспериментов, включающих анализ как естественной конвекции, так и вынужденного течения. Проанализировано влияние типа течения и граничных условий на характеристики теплообмена и формирование температурных полей в системе.

Ключевые слова: ICFD, численное моделирование, сопряженный теплообмен, естественная конвекция, вынужденное течение, LS-DYNA

Numerical simulation of heat and mass transfer in the air gap between plates using the ICFD method

Abstract. This paper presents the results of a numerical study of heat transfer in an air gap between two parallel copper plates. The lower plate is heated, while the upper plate is cooled by convection. The study was performed using the LS-DYNA software package and the ICFD (Incompressible Computational Fluid Dynamics) method for simulating air flow and heat transfer. The problem statement and mathematical model are presented. Primary attention is paid to the description of the ICFD method and its key algorithms for solving the Navier-Stokes equations for an incompressible fluid. A series of computational experiments were conducted, including the analysis of both natural convection and forced flow. The influence of the flow type and boundary conditions on the heat transfer characteristics and the formation of temperature fields in the system is analyzed.

Keywords: ICFD, numerical simulation, conjugate heat transfer, natural convection, forced flow, LS-DYNA

Список источников

[1] 	ICFD THEORY MANUAL. Incompressible fluid solver in LS-DYNA // URL: https://ftp.lstc.com/anonymous/outgoing/inaki/docs/pdf_icfd/ICFD_theory.pdf (дата обращения: 07.11.2025).

[2] 	A. Chorin, Numerical solution of the navier-stokes equations, Math Comput, 22 (1968)

[3] 	R. Lohner, C. Yang, J. Cebral, J. Baum, H. Luo, D. Pelessone, and C. C., Fluid-structure interaction using a loose coupling algorithm and adaptive unstructured grids., AIAA, 95 (1995).

[4] 	F. D. Pin, The Meshless Finite Element Method Applied to a Lagrangian Particle Formulation of Fluid Flows, PhD thesis, Facultad de Ingenieria y Ciencias Hidricas (FICH) Instituto de Desarrollo Tecnologico para la Industria Quimica (INTEC) Universidad Nacional del Litoral, 2003.

[5] 	S. Idelsohn, F. D. Pin, R. Rossi, and E. Onate, Avoiding the instabilities caused by added mass effects in fluid structure interaction problems for pressure segregation and staggered approaches, Int. J. Numer. Meth. Engng (2009), (2009).