УДК 531

Особые управления первого порядка для задачи минимизации времени двухзвенного манипулятора

Обрадович Александр Миленкович1 		aobradovic@mas.bg.ac.rs

Черкасов Олег Юрьевич2,3 , Селюцкий Юрий Дмитриевич4

1 Факультет машиностроения Белградского университета, Белград, Сербия

2 Университет МГУ-ППИ в Шэньчжэне

3 Механико-математический факультет МГУ, Москва, Россия

4 НИИ механики МГУ, Москва, Россия

Аннотация. Анализируется задача минимизации времени движения двухзвенного манипулятора в горизонтальной плоскости [1, 2]. Применение принципа максимума Понтрягина позволяет установить, что возможны три типа управления: регулярное управление, принимающее граничные значения («банг-банг») для силы и момента, сингулярное (особое [3]) управление первого порядка для момента и сингулярное управление второго порядка для силы . В статье [1] приведены примеры крайних положений, в которых управление силой и моментом имеет вид «банг-банг» за весь временной интервал. Дано объяснение с точки зрения механики сингулярного управления второго порядка для силы. Показано также [1] , что такой тип решения в данном случае может быть сопряжен только с режимом «четтеринга». Исследована возможность существования особых управлений первого порядка для момента, удовлетворяющих условиям Келли [4] и условиям сопряжения особых и неособых участков [5].

Ключевые слова: минимизации времени, двухзвенний манипулятор, особые управления первого порядка, условия Келли, условия сопряжения особых и неособых участков

First-order singular controls for the time minimization problem of a two-link manipulator

Obradović Aleksandar Milenkovich1 				aobradovic@mas.bg.ac.rs

Cherkasov Oleg Yuryevich2,3 , Selyutskiy Yuriy Dmitrievich4

1 Faculty of Mechanical Engineering, University of Belgrade, Belgrade, Serbia

2 Shenzhen MSU-BIT University

3 Faculty of Mechanics and Mathematics, Moscow State University, Moscow, Russia

4 Research Institute of Mechanics, Moscow State University, Moscow, Russia

Abstract. The problem of minimizing the time of motion of a two-link manipulator in the horizontal plane is analyzed [1, 2]. Application of Pontryagin's maximum principle allows us to establish that three types of control are possible: regular control taking boundary values ​​("bang-bang") for force and torque, singular  [3] first-order control for torque, and singular second-order control for force. The article [1] provides examples of  situations in which the control of force and torque has the "bang-bang" form over the entire time interval. An explanation is given from the mechanics point of view of singular second-order control for force. It is shown [1] that second-order control for force  in this case can only be associated with the "chattering" regime. Here, the possibility of the existence of singular first-order controls for torque, that satisfy the Kelly conditions [4] and the conditions for the conjugation of singular and non-singular sections [5], is investigated.

Keywords: time minimization, two-link manipulator, first-order singular controls, Kelly conditions, conditions for the conjugation of singular and non-singular sections

Список источников

[1] 	Осипов С.Н., Формальский A.M. Задача о быстрейшем повороте манипулятора // ПММ. 1988. Т. 52, вып. 6. С.  29-933

[2] 	Obradović A.: Optimal control of a rigid body motion in a complex case// TAM. 1991. V. 17, No. 1. P.  121-132.

[3] 	Габасов Р., Кириллова Ф.М. Особые оптимальные управления. URSS.2018. 256 с.

[4] 	Kelley H.J., Kopp R.E., Moyer H.G., Leitmann G. (Ed.) Singular Extremals. Topics in Optimization. // Academic Press, New York 1967. P. 63-101.

[5] 	Odia A., Bell D.J.: Junction point on partially singular trajectories. // Int. J. Control. 2012. V. 85. P. 1996–2003.5.141