Влияние интенсивности начальной закрутки на расширение потока влажного воздуха в сверхзвуковом сопле при наличии спонтанной конденсации

А.Г. Здитовец*, Н.А. Киселёв, Ю.А. Виноградов

НИИ механики МГУ, Москва, Россия

*E-mail (докладчика): zditovets@mail.ru

Аннотация. 	Расширение влажного воздуха в сверхзвуковых соплах сопровождается спонтанной конденсацией паров воды из-за резкого снижения температуры потока. Однако в пограничном слое из-за вязкой диссипации температура потока близка к начальной и, следовательно, в пристенной области водяной пар остается в перегретом состоянии и не конденсируется. Если капли конденсата проникают в эту область, они испаряются и охлаждают ее. Как результат температура стенки должна значительно снижаться вплоть до локальной статической температуры потока. В работе [1] экспериментально показано, что в области за скачком конденсации температура стенки сопла может быть как больше, так и меньше величины характерной для однофазного течения при идентичных начальных параметрах потока, что, вероятно, связано с ограниченной скоростью проникновения капель конденсата в пограничный слой, определяемой турбулентной диффузией. Один из способов интенсифицировать этот процесс – это придать потоку начальную закрутку. В настоящей работе приведены результаты экспериментального исследования влияния интенсивности начальной закрутки на температуру стенки при расширении влажного воздуха до сверхзвуковой скорости.

Ключевые слова: спонтанная конденсация, адиабатная температура стенки

.

Исследование выполнено в рамках государственного задания МГУ имени М.В. Ломоносова

Литература

1. 	Zditovets A.G., Kiselev N., Vinogradov Y.A., Popovich S. Adiabatic wall temperature in the supersonic flow of moist air with spontaneous condensation // Experimental Thermal and Fluid Science. 2024. № September 2023(150). С. 111057. DOI:10.1016/j.expthermflusci.2023.111057.

Effect of Swirl Intensity on Humid Air Expansion with Spontaneous Condensation in a Supersonic Nozzle

A.G. Zditovets*, N.A. Kiselev, Yu.A. Vinogradov

Lomonosov Moscow State University, Institute of Mechanics, Moscow, Russia

* E-mail (zditovets@mail.ru):

Annotation. The expansion of humid air in supersonic nozzles is accompanied by spontaneous condensation of water vapour due to the sharp drop in flow temperature. In the boundary layer, however, viscous dissipation maintains the gas temperature close to its initial value, so the vapour remains superheated and does not condense near the wall. If condensate droplets penetrate this region, they evaporate and cool it, and the wall temperature may decrease down to the local static temperature. Experiments in [1] showed that, downstream of the condensation front, the wall temperature may be either higher or lower than in single-phase flow with identical inlet conditions. This behaviour is presumably linked to the limited rate of droplet penetration into the boundary layer governed by turbulent diffusion. One way to intensify this process is to impart initial swirl to the flow. The present work reports an experimental study of how the intensity of inlet swirl affects the wall temperature during the supersonic expansion of humid air.

Keywords: spontaneous condensation, adiabatic wall temperature

The study was conducted under the state assignment of Lomonosov Moscow University.

References

1. 	Zditovets A.G., Kiselev N., Vinogradov Y.A., Popovich S. Adiabatic wall temperature in the supersonic flow of moist air with spontaneous condensation // Experimental Thermal and Fluid Science. 2024. № September 2023(150). С. 111057. DOI:10.1016/j.expthermflusci.2023.111057.