УДК 681.5.01:531.53

О максимизации области управляемости двузвенного перевернутого маятника на качелях seesaw при согласовании управлений в шарнирах

Кручинин Павел Анатольевич1		pkruch@mech.math.msu.su

SPIN-код: 3265-0739

Алексеева Надежда Валерьевна 1(*)

1 МГУ им. М.В. Ломоносова, Москва, Россия

Аннотация. Рассматриваемая задача  математического моделирования процесса сагиттальных колебаний человека, стоящего на качелях seesaw c прямыми ногами. Модель представляет собой плоский двойной перевёрнутый маятник, установленный на seesaw. Нижний цилиндрический шарнир моделирует оба голеностопных сустава человека, а межзвенный  шарнир – оба тазобедренных. суставы человека. Качели представляют собой сегмент цилиндра, ось которого перпендикулярна плоскости движения маятника. Описываемая система имеет три степени свободы и в отсутствие управления имеет неустойчивое положение равновесия, в котором оба звена маятника направлены вертикально вверх, а качели не наклонены. Управлением являются моменты, прикладываемые в осях шарниров согласованно. То есть для моделирования мышечной синергии считается, что моменты в шарнирах не являются независимыми и выражаются через общее управление,  которое ограничено по абсолютной величине. Таким образом описывается ситуация, аналогичная мышечной синергии. Рассмотрены свойства областей управляемости при изменении распределения моментов в шарнирах. Найдены ситуации, в которых размер области управляемости принимает максимальные и минимальные значения.

Ключевые слова: стабилизация, двузвенный маятник, перевернутый маятник, позовый контроль, область управляемости

On maximizing the controllability region of a two-link inverted pendulum on a seesaw with coordinated controls in the hinges

Kruchinin Pavel Anatolievich1			pkruch@mech.math.msu.su

SPIN-код: 3265-0739

Alexeeva Nadezhda Valerievna1(*)

1 Lomonosov Moscow State University, Moscow, Russia

Abstract. The problem under consideration is the mathematical modeling of the sagittal oscillations of a person standing on a seesaw with straight legs. The model is a double inverted pendulum mounted on a seesaw. The lower cylindrical hinge models both ankle joints, and the interlink hinge models both hip joints. The seesaw is a segment of a cylinder whose axis is perpendicular to the pendulum plane. The described system has three degrees of freedom and, in the absence of control, has an unstable equilibrium position in which both pendulum links point vertically upward, and the swing is not tilted. The control is provided by moments applied in a coordinated fashion to the hinge axes. That is, to model muscle synergy, it is assumed that the moments in the hinges are not independent and are expressed through a common control, which is limited in absolute value. This describes a situation analogous to muscle synergy. The properties of the control regions are considered when the distribution of moments in the hinges varies. Situations were found in which the size of the controllability region takes on maximum and minimum values..

Keywords: stabilisation, two-link pendulum, inverted pendulum, postural control, controllability region