EXPERIMENTAL AND NUMERICAL ANALYSIS OF FLOW STRUCTURES IN CYLINDRICAL CAVITIES UNDER SUBSONIC FLOW IN A FLAT CHANNEL

M.A. Zubin, A.F. Zubkov, A.Yu. Chulyunin

Lomonosov Moscow State University, Research Institute of Mechanics, Moscow, Russia

e-mail: zubinma@mail.ru, e-mail: 9392998@mail.ru, e-mail: chulyu-n@mail.ru

The results of experimental and numerical investigations of turbulent subsonic flow past an inclined oval-trench dimple (OTD) [1] are presented. The OTD is a finite-span cylindrical trench with rounded spherical ends, whose longitudinal axis is oriented at an acute angle to the incoming flow. A modified configuration—the V-cavity—was also studied. The latter is formed by two smoothly connected halves of inclined OTDs arranged symmetrically with respect to the oncoming flow, thus exhibiting a V-shaped form in a plane [2].

Distributions of the pressure coefficient were obtained, and surface streamline patterns were visualized for the cylindrical cavities with various relative depths. A comparative analysis was carried out for the separated recirculating flow inside the inclined OTD and the V-cavity, which, as established in [1], governs the heat-transfer characteristics of surfaces with artificial reliefs.

Comparison of the surface pressure data and streamline patterns demonstrates that the mechanisms of longitudinal vortex flow deceleration in the upstream half of the OTD and in each wing of the V-cavity are essentially identical. It is shown that the region of interaction between the counter-rotating vortical flows in the vicinity of the symmetry plane does not significantly affect the flow structure within the cavity wings. This indicates the advantage of V-shaped oval-trench cavities, where enhanced recirculation is observed in both branches inclined at 45º to the oncoming flow, in contrast to a classical OTD with the same angle of inclination [2].

Comparison of the experimental results with RANS-based numerical simulations demonstrates that the proposed approach can be effectively applied in the search for optimal shapes of artificial reliefs for heat-transfer surfaces.

The study was conducted under the state assignment of Lomonosov Moscow State University.

Keywords: turbulent flow, separation flows, flow past a cavity, pressure coefficient

References

1. Isaev S.A. Genesis of Anomalous Intensification of Separation Flow and Heat Transfer in Inclined Grooves on Structured Surfaces // Fluid Dynamics. 2022. Vol. 57. No. 5. P. 558–570.

2. Zubin M. A., Zubkov A. F., Chulyunin A. Yu. A.  Structure of Subsonic Separation Flow past Cylindrical Cavities of Various Shapes in Plan on the Wall of a Plane Channel // Fluid Dynamics. 2025. Vol. 60:29.