Кинематика лабораторного встряхивателя на базе планетарного механизма с некруглыми зубчатыми колесами

Азизов Э.Ю., Приходько А.А.

Кубанский государственный технологический университет, Краснодар, Россия

E-mail: azizoved@gmail.com

Исследуется кинематика механизма с возвратно-вращательным движением выходного звена, представляющего собой двухрядную планетарную передачу с внутренним зацеплением. Каждая пара зубчатых колес состоит из эллиптического колеса с внешними зубьями, ось вращения которого расположена в фокусе делительного эллипса, и некруглого зубчатого колеса с внутренними зубьями. Построена центроида некруглого колеса из условия равенства скоростей в точке зацепления, при этом за цикл работы механизма эллиптическое зубчатое колесо совершает три оборота вокруг своей оси. Проведен кинематический анализ предложенной планетарной передачи, в результате которого построена функция угла поворота и определены углы качания выходного вала при различных эксцентриситетах эллиптического зубчатого колеса. Предлагаемый механизм может быть использован в составе привода лабораторного встряхивателя с возвратно-вращательным движением пробирок.

Ключевые слова: возвратно-вращательное движение, планетарный механизм, кинематический анализ.

Kinematics of a laboratory shaker based on a planetary mechanism with non-circular gears

Azizov E.Yu., Prikhodko A.A.

Kuban State Technological University, Krasnodar, Russia

E-mail: azizoved@gmail.com

The paper investigates kinematics of a mechanism with a reciprocating rotational motion of the output link, which is a two-row planetary gear with internal gearing. Each pair of gears consists of an elliptical gear with external teeth, the axis of rotation of which is located at the focus of the pitch ellipse, and a non-circular gear with internal teeth. The centroid of a non-circular wheel is constructed from the condition of equality of velocities at the point of engagement, while the elliptical gear makes three revolutions around its axis during the cycle of the mechanism. A kinematic analysis of the proposed planetary gear was carried out, as a result of which the function of the angle of rotation was constructed and the swing angles of the output shaft were determined for various eccentricities of the elliptical gear. The proposed mechanism can be used as part of a drive for a laboratory shaker with a reciprocating rotational movement of test tubes.

Key words: rotationally reciprocating motion, planetary gear, kinematic analysis.