Задача оптимального торможения центра масс спускаемого летательного аппарата в сопротивляющейся среде

Н.А. Орёл1*

1МГУ им. М.В. Ломоносова, Москва, Россия
*E-mail: nikita.orel@math.msu.ru

Аннотация

В работе будет рассмотрена задача оптимального управления процессом торможения спускаемого летательного аппарата в сопротивляющейся среде при движении в вертикальной плоскости. Модель предполагает пренебрежимое влияние расхода топлива на движение центра масс. В качестве управляющих переменных используются тяга и угол наклона траектории, а целевым функционалом является квадрат скорости аппарата в заданный конечный момент времени. С использованием принципа максимума Понтрягина исходная задача оптимального управления сводится к краевой задаче для системы нелинейных дифференциальных уравнений. Устанавливается неоптимальность особых режимов управления в случае линейного сопротивления. Анализ с использованием первого интеграла системы позволяет определить возможное количество переключений тяги вдоль оптимальной траектории и получить достаточные условия для случая, когда возможно не более одного переключения. Полученные качественные результаты проиллюстрированы численным моделированием.

Ключевые слова: оптимальное торможение, принцип максимума, особое управление, фазовый портрет

Литература

- J. Meditch, On the problem of optimal thrust programming for a lunar soft landing. IEEE Transactions on Automatic Control, vol. 9, pp. 477–484, Oct. 1964.

- Topcu U., Casoliva J., Kenneth D. Mease. Minimum-Fuel Powered Descent for Mars Pinpoint Landing., Journal of Spacecraft and Rockets, 44(2), 324-331, 2007.

- Понтрягин Л.С., Болтянский В.Г., Гамкрелидзе Р.В., Мищенко Е.Ф. Математическая теория оптимальных процессов. М.: Наука, 1983, 393 с.

- H.J. Kelley, R.E. Kopp, and H.G. Moyer. Singular extremals. In Topics in Optimization, pp 63–101. Academic Press, New York, 1967.

The problem of optimal braking of a lander’s mass center in a resisting medium

N.A. Oryol1*

1Lomonosov Moscow State University, Moscow, Russia

* E-mail: nikita.orel@math.msu.ru

Аnnotation

The problem of optimal control for the braking process of a lander in a resisting medium during motion in the vertical plane is considered. The model assumes a negligible influence of fuel consumption on the center of mass motion. The thrust and the flight path angle are used as control variables, and the terminal functional is the square of the vehicle's velocity at a specified final time moment. Using the Pontryagin maximum principle, the optimal control problem is reduced to a boundary value problem for a system of nonlinear differential equations. The non-optimality of singular control for a linear resisting medium is established. Analysis using a first integral of the system allows for determining the possible number of thrust switchings along the optimal trajectory and obtaining sufficient conditions for the case where no more than one switching is possible. The obtained qualitative results are illustrated by numerical simulation.

Keywords: optimal braking, maximum principle, singular control, phase portrait

References

- J. Meditch, On the problem of optimal thrust programming for a lunar soft landing. IEEE Transactions on Automatic Control, vol. 9, pp. 477–484, Oct. 1964.

- Topcu U., Casoliva J., Kenneth D. Mease. Minimum-Fuel Powered Descent for Mars Pinpoint Landing., Journal of Spacecraft and Rockets, 44(2), 324-331, 2007.

- Pontryagin, L.S., Boltyansky V.G., Gamkrelidze R.V., Mishchenko E.F. Matematicheskaya teoriya optimalnykh protsessov. Moscow, Nauka Publ., 1983, 393 p.

- H.J. Kelley, R.E. Kopp, and H.G. Moyer. Singular extremals. In Topics in Optimization, pp 63–101. Academic Press, New York, 1967.