Связь высокочастотных автоколебаний в генераторе импульсных струй с Релей-Тейлоровской неустойчивостью границы каверны.

С.А. Очеретяный1, В.В. Прокофьев1, Г.В. Топейцев1

1Институт механики МГУ, Москва

vlad.prokof@yandex.ru

Исследуются режимы работы генератора импульсных струй, состоящего из кавитатора, вентилируемой каверны, соединительного канала и сужающегося сопла на выходе. Основное внимание в докладе уделено высокочастотному режиму автоколебаний, когда колебания давления в каверне и в форкамере практически отсутствуют, но имеет место прерывистое истечение жидкости из сопла.

Предположение, что такие автоколебания могут быть связаны с Релей-Тейлоровской неустойчивостью границы струи подкреплено анализом точного решения плоской задачи о взаимодействии конечной струи с наклонной пластиной при различных давлениях на поверхностях падающей и истекающей струй. Задача решена точно методами ТФКП с помощью квазидвоякопериодических тэта функций. Показано, что течение с неустойчивой границей струи имеет место вблизи предельного решения, когда струя замыкается без возвратной струи на поверхности наклонной пластины. Получена связь между числом кавитации предельного течения и параметрами генератора импульсных струй.

Ключевые слова: кавитация, автоколебания, отрицательное число кавитации, неустойчивость Релея – Тейлора, генератор импульсных струй.

On the Relationship Between High-Frequency Self-Oscillations in a Pulsed-Jet Generator and the Rayleigh–Taylor Instability of the Cavity Boundary

S.A. Ocheretyanyi, V.V. Prokofiev, G.V. Topeitsev

Institute of Mechanics, Lomonosov Moscow State University, Moscow, Russia

Operating regimes of a pulsed-jet generator composed of a cavitator, a ventilated cavity, a connecting channel, and a converging nozzle at the outlet are investigated. Particular attention is given to a high-frequency self-oscillation regime, in which pressure oscillations in the cavity and the plenum chamber are negligible, while intermittent fluid ejection from the nozzle is observed.

It is hypothesized that such self-oscillations are associated with the Rayleigh–Taylor instability of the jet boundary. This assumption is supported by an analysis of an exact solution to the two-dimensional problem of interaction between a finite jet and an inclined plate under different pressures on the surfaces of the incident and effluent jets. The problem is solved exactly by methods of complex analysis (theory of functions of a complex variable) using quasi–doubly periodic theta functions. It is shown that a flow with an unstable jet boundary arises in the vicinity of the limiting solution, when the jet closes without a re-entrant jet on the surface of the inclined plate. A relation is obtained between the cavitation number of the limiting flow and the parameters of the pulsed-jet generator.

Keywords: cavitation; self-oscillations; Rayleigh–Taylor instability; pulsed-jet generator; ventilated cavity; negative cavitation number