СТРУКТУРА ТЕЧЕНИЯ ОКОЛО ПОЛУКОНУСА

НА ПЛАСТИНЕ ПРИ ОБТЕКАНИИ СВЕРХЗВУКОВЫМ ПОТОКОМ

М.А. Зубин,  Ф.А. Максимов

МГУ им. М.В. Ломоносова, Научно-исследовательский институт механики, Москва, Россия

e-mail: zubinma@mail.ru,  e-mail: f_a_maximov@mail.ru

Представлены результаты экспериментального и численного исследований структуры сверхзвукового (М=3) обтекания компоновки полуконус на пластине, вершина которого совпадает с прямой сверхзвуковой кромкой пластины. Экспериментально с помощью специального оптического метода для визуализации сверхзвуковых конических течений установлено [1], что область отрыва, возникающая при взаимодействии ударной волны с пограничным слоем на пластине, целиком располагается на пластине. Выяснено, что линия растекания на поверхности полуконуса не является, вопреки имеющемуся в литературе представлению [2, 3], линией присоединения оторвавшегося на пластине потока. Установлено, что структура течения в ударном слое определяется интенсивностью контактных разрывов, исходящих из точек ветвления сопутствующей ударно-волновой конфигурации. При этом в ударном слое могут существовать либо стандартные особенности Ферри на поверхности полуконуса, либо вихревые над его поверхностью. На основе моделей идеального и вязкого газа разработаны численные коды для расчета течения в коническом приближении. Сравнение результатов расчетов с экспериментальными данными показало их удовлетворительное согласие и границы применения каждого из них.

Исследование выполнено в рамках государственного задания МГУ имени М.В. Ломоносова.

Ключевые слова: сверхзвуковые конические течения, ударные волны, контактный разрыв, отрыв турбулентного пограничного слоя, особенность Ферри.

Литература

1. Зубин М.А., Максимов Ф.А. О некоторых особенностях течения в ударном слое около полуконуса на пластине // Изв. РАН. МЖГ. 2024. № 5. С.107–119.

2. Авдуевский В.С., Грецов В.К. Исследование трехмерного отрывного обтекания полуконусов, установленных на пластине // Изв. АН СССР. МЖГ. 1970. №6. С. 112-115.

3. Settles G.S., Kimmel R.L. Similarity of quasiconical shock wave/turbulent boundary layer interactions // AIAA Journal. 1986. Vol. 24. No. 1. pp. 47-53.

FLOW STRUCTURE AROUND A HALF-CONE ON A PLATE IN SUPERSONIC CURRENT

M.A. Zubin, F.A. Maksimov

Lomonosov Moscow State University, Research Institute of Mechanics, Moscow, Russia

e-mail: zubinma@mail.ru, e-mail: f_a_maximov@mail.ru

The article presents the results of experimental and numerical studies of the structure of supersonic (M=3) flow past a semi-cone assembly on a plate, the apex of which coincides with the straight supersonic edge of the plate. Using a specialized optical method for visualizing supersonic conical flows, it was experimentally established [1] that the separation region arising from the interaction of a shock wave with the boundary layer on the plate is located entirely on the plate. It was found that the spreading line on the surface of the semi-cone is not, contrary to the existing idea in the literature [2, 3], the reattachment line of the flow separated on the plate. It was established that the flow structure in the shock layer is determined by the intensity of contact discontinuities emanating from the branching points of the accompanying shock-wave configuration. In this case, either standard Ferry singularities on the surface of the semi-cone or vortex singularities above its surface can exist in the shock layer. Numerical codes for flow calculations in the conical approximation were developed based on ideal and viscous gas models. Comparison of the calculated results with experimental data demonstrated satisfactory agreement and the applicability limits of each.

The study was conducted under the state assignment of Lomonosov Moscow State University.

Keywords: supersonic conical flows, shock waves, contact discontinuity, turbulent boundary layer separation, Ferry singularity.

References

1. Zubin M.A., Maksimov F.A. Specific Features of the Flow in the Shock Layer near a Semicone on a Flat Plate // Fluid Dynamics. 2024. Vol. 59. No. 5. P. 1117–1128.

2. Avduevsky V.S., Gretsov V.K. A study of three-dimensional separated flow past semi-cones mounted on a plate // Izvestiya AN SSSR. MZhG. 1970. No. 6. P. 112-115.

3. Settles G.S., Kimmel R.L. Similarity of quasiconical shock wave/turbulent boundary layer interactions // AIAA Journal. 1986. Vol. 24. No. 1. P. 47-53.